MODIFIED  PRODUCTION  RULES

This class is for 1955-1974 street driven musclecars, which have minor modifications to the engine, transmission, etc. Unless it is specifically stated herein, only factory issued or factory service replacement parts from that era will be allowed.

Air cleaner: Stock or open aftermarket allowed. Element diameter not to exceed 14", height not to exceed 3".

Accessories: Must be operational. Power steering belts must be tight. A/C belts may be removed.

Alternator: Required, and operating.

Battery: Stock location, as delivered from factory.

Block: Must be same as body manufacturer.

Body: Must have complete stock factory appearance. Dealer built package cars allowed (Yenko, Harrel, Motion, Mr. Norm, etc.). Rear wheel wells may have rolled lips only, no radiusing allowed. Front inner fender wells required.

Brakes: Must be OEM complete, upgrades from front drum to OEM front disc allowed. Line locks allowed.

Bumpers, lights and grille: Must be OEM complete, and correct for year body claimed.

Camshaft: Hydraulic, solid flat tappet or roller allowed, see Engine Rules. Engine must pull a minimum of 12" vacuum at 1200 rpm.

Carburetion: Must appear OEM complete for year, make and model of specified engine. Stock metal fuel lines required. Carb must resemble what was offered from the factory for engine package claimed. Exact part numbers not required. Factory dual carb's, tri-powers, cross rams allowed. Holley's with vacuum secondaries may use any vacuum model that resembles the factory piece. Original GM Rochester Q-jets may use current factory, Weber or Edelbrock exact replacements. Mopars may use Carter or Edelbrock Comp. AFB series, or Thunder Series AVS replacements. No double pump Holley carbs unless factory installed. Dealer/factory cars with double pump or multiple carbs must show proof of such.

Class designations: MP for all participants, followed by 3 digit entry number.

Conduct: Every participant and spectator is expected to conduct themselves in a professional, sportsmanlike, and respectful manner. Anyone violating this rule will be expelled for life. There will be NO first warnings.

Cylinder heads: Any OEM head that resembles the factory parts for engine claimed are allowed. No aftermarket heads are allowed, aluminum or steel. Heads must be representative of the year body & engine package claimed.

Clutch: Explosion resistant clutches strongly encouraged. Explosion proof bellhousings required at 11.49 E.T.

Dealer or factory built cars: Participation of these cars is strongly encouraged, within the rules outlined herein.

Distributor: OEM, or OEM replacement distributor which exactly resembles the factory unit allowed. Stock style caps mandatory. Electronic conversions allowed, but electronic control boxes must be hidden. No H.E.I.. Rev limiters encouraged. Stock style 8mm black wires only, no lettering, with factory looms.

Drive shaft loop: Required for all entrants.

Engine: Must be the same make as body, and exactly resemble factory built. No small block to big block conversions allowed, unless the factory offered big blocks in the year body claimed. Any factory exhaust/intake manifold/block/head allowed on any year combo. Factory aluminum intakes and factory tri-powers allowed. Random P&G inspections will occur unannounced. Factory style water pumps required. OEM valve covers mandatory, with maximum of 2 gaskets. No valve cover spacers. Single 1/2" carb spacer allowed. Stock style pulleys required, must be black or engine color. Superceded casting numbers allowed with factory catalog proof. Roller cams/rockers allowed. Engine must APPEAR to be factory built. Superceded parts are limited to blocks, intakes, carb, and cylinder heads, and then only where the OEM manufacturer shows those parts listed in their catalog. Crate engines allowed, as long as all other rules hereien are applicable.

Engine size: 500" max. Factory manifolds required between 476" to 500". We will P&G the top 5 cars. Any violators will be permanently banned.

Exhaust system: 3" max diameter from header collector reducer, or factory manifold outlet, through to stock exit tailpipes. Oval mufflers required, minimum 14" case length. X and H pipes allowed. Tail pipes must exit in factory stock location.

Fan & fan clutch: Factory steel fan blade only. Fan clutch required, if factory issued.

Fuel pumps: Factory style mechanical fuel pump required, and functional. Electric pumps allowed, but must be hidden from view, and on/off switchable with ignition only. Stock fuel lines only, no rubber allowed, unless factory issued.

Fuel: Gasoline only, any octane. No additives. No nitrous.

Gauges: No external or engine compartment gauges allowed. Dash or column mounted tachs allowed, as well as neatly mounted underdash gauges. No dash top gauges other than single tach allowed.

Headers: Allowed, but not required, see engine spec's. No fenderwell headers allowed however. No underchassis open exhaust cutouts allowed.

Helmet: SN90 or newer. Helmet usage is mandatory for all participants.

Hood: Stock factory hood only, unless proof of different dealer installed package is furnished.

Hood scoops: Factory hoods only, unless proof of dealer/factory installed package is furnished.

Intake manifold: Only factory intakes are allowed, either cast iron or aluminum. Must be consistent with factory engine package claimed.

Interior: Full factory appearance interior required.

Pollution controls: May be removed.

Race format: Qualifying will be based on the AVERAGE  of the best 2 runs of 3. Race will be run on best 2 of 3 format, with closest qualifiers racing each other. NO sandbagging. If you run quicker in eliminations than .10 of your fastest time in qualifying, you will be disqualified. IE; you run 11.90, 12.00, 12.10..then your average is 12.00, you cannot run faster than 11.80 in eliminations. Your permanent car number is the last four digits of your Social Security number.

Radiators: Stock style only. No aluminum radiators, unless factory installed.

Rear axle: Must be the same as corporate OEM body. Any gear and spools allowed.

Roll bars: Mandatory at 11.49 for all cars.

Safety: Full compliance of current year I.H.R.A Safety Rules is expected.  Specific safety scrutiny will start at 11.49 E.T., this applies to every participant, no exceptions.

Seat belts: Complete set of OEM seat belts required. Shoulder belt usage required if factory equipped. Faster than 11.49 will revert to I.H.R.A. seatbelt rules.

Street equipment: All factory equipment must be installed and functional. Car must be current year licensed and registered, and proof must be furnished at each event tech, to comply with the 15 mile cruise requirement.

Superchargers: Allowed only on those cars where they were a factory or dealer package installed option. They will only be allowed on the exact year body claimed. No exceptions. Proof must be furnished.

Suspension: OEM in appearance. Aftermarket stock style springs and shocks allowed. Traction devices limited to bolt on snubbers, slapper bars, boxed upper and lower control arms, etc. Front sway bar removal allowed. Subframe connectors allowed. Roll bar, if installed, MUST be welded to the subframe connector.

Tires, front: Must be OEM diameter and footprint for body style claimed. Bias ply or radials allowed. Matching brands front to rear encouraged. Front and rear tires must be of the same construction.

Tires, rear: Bias ply repro tires, current radials, or any D.O.T. tire up to approx. 28.0" diameter and 9.5" footprint, not exceeding 275-60-15 description. A drive tire spec chart will follow later.

Transmission: Must be the same as the OEM factory offered with the claimed engine package. 3500 max stall convertor.

Wheels: Factory stock, or *period correct* for the 1955-1974 era are required. Must be the same diameter front and rear. All four wheels must match, unless proof of a different factory/dealer installed package is furnished. Minimum of 5.5" front wheel width, and maximum 8" rear wheel width, measured at the bead area.